# Automated Test Environment Design for Complex Radar Systems

## 1. System Topology & Hardware Overview

- Unit Under Test (UUT):
The radar device transmits and receives data, performs internal processing, and communicates with a Host PC via Ethernet (ETH).

- Host Environment:
A PC running a dedicated driver library that exposes functional APIs to interact with the radar.

- Peripheral Equipment:

  - Power Supply Unit (PSU): Software-controlled, providing power to the radar.

  - PTP Time Synchronization: Managed by the Host; the radar requires valid PTP sync to operate.

- Test Types:

  - Functional

  - Durability

  - New feature integration

  - Full regression

## 2. Codebase & Git Strategy

- Test Code Repository:

  - Modular, generic repository based on Pytest.

  - Test code is built from atomic "actions" (functions) that represent radar operations.

  - Maintained in sync with requirements documentation.

- Embedded Code Repository:

  - Contains the radar’s embedded code.

  - Dedicated pipelines for daily (nightly) and official (bi-weekly) builds.

- Configuration & Limits Repository:

  - Stores configuration and threshold files.

  - Full versioning support to ensure backward compatibility for customers using older versions.

## 3. Jira Xray Integration (Test Targeting & Reporting)

- Jira as Source of Truth:

  - Requirements are covered by Test Templates in Xray.

  - Tests are organized into Test Sets (e.g., "Sanity", "Regression_v2.1", "Customer_X_Acceptance").

- Test Fetching (GitLab to Jira):

  - At pipeline start, the system queries the Xray API to fetch the list of Test IDs from the specified Test Set.

- Mapping:

  - Each Test ID from Jira is mapped to a corresponding Pytest function (using markers or custom fields).

- Reporting (Jira Test Execution):

  - After execution, the pipeline sends a results file (JSON/XML) back to Xray.

  - The integration automatically creates a Test Execution issue, updates the pass/fail status for each template, and attaches a detailed report (Allure/HTML).

## 4. Resource Management & Job Allocation

- Queuing & Resource Manager:

  - The environment manages a job queue and allocates jobs to available test benches based on required hardware type.

- Job Parameters:

  - Each job is dispatched with a "package" containing:

    - Build version

    - Hardware type

    - Configuration file

    - Relevant threshold file

- Triggering:

  - Supports both automatic triggering (post-build) and manual triggering by authorized users.

## 5. Output & Reporting

- Clear Reports:

  - Reports are separated by test type and hardware type.

- Detailed Error Tracebacks:

  - Deep error tracebacks are provided for quick debugging, especially for latency or data processing failures.

- Log Attachment:

  - API logs and PSU/PTP data are attached to each failure for comprehensive analysis.

## 6. High-Level Architecture Diagram

- [GitLab CI/CD]

- |

- v

- [Resource Manager] <----> [Test Benches Pool]

- |                          |

- v                          v

- [Jira Xray API]           [Host PC + UUT + PSU/PTP]

- |

- v

- [Reporting & Dashboard]GitLab CI/CD: Orchestrates the entire process, including both automatic and manual triggers.

- Resource Manager: Handles job queuing, allocation, and monitoring of test benches.

- Test Benches Pool: Physical stations with various hardware, managed by the Resource Manager.

- Jira Xray API: Used for fetching test sets, mapping, and reporting results.

- Reporting: Generates detailed reports, logs, and dashboards.

## 7. Key Design Principles

- Modularity & Flexibility:

  - Test code is decoupled from embedded code for easy maintenance and scalability.

  - Atomic actions enable reusable and extensible test logic.

  - All parameters (version, hardware, thresholds) are managed externally via configuration files.

- Bidirectional Jira Xray Integration:

  - Dynamic fetching of test sets via API.

  - Automated mapping and result reporting.

- Smart Resource Management:

  - Centralized job queue and allocation logic.

  - Scalable to support additional hardware/test benches.

- Comprehensive Reporting:

  - Allure/HTML reports, error tracebacks, and log attachments for every test run.

## 8. Example Workflow

- Trigger:

  - Automatic (post-build) or manual (user-initiated) trigger in GitLab.

- Test Set Fetch:

  - Fetch the relevant Test Set from Jira Xray via API.

- Test Mapping:

  - Map Test IDs to Pytest functions.

- Resource Allocation:

  - Resource Manager assigns the job to an available test bench with the required hardware.

- Test Execution:

  - Tests run with the specified configuration and thresholds.

- Result Collection:

  - Gather results, logs, and tracebacks.

- Reporting:

  - Send results to Jira Xray (Test Execution), generate Allure/HTML reports, and update dashboards.

## 9. Next Steps

- Define the detailed design and implementation plan for the Resource Manager (parameters, prioritization, monitoring).

- Specify the configuration and threshold file formats (e.g., JSON/YAML).

- Choose and implement the mapping method between Test IDs and Pytest functions.

- Set up the Xray integration for fetching, mapping, and reporting.

- Develop a Proof of Concept (PoC) to validate the end-to-end workflow.

## 10. References & Resources

- Xray Test Management REST API Documentation

- Pytest + Allure Integration Guide

- GitLab CI/CD Pipelines Documentation